Motorway traffic noise reduction by means of barriers:

a design example based on prediction models

and experimental verification.

Angelo Farina (*), Patrizio Fausti (**)

(*)	Dipartimento di Ingegneria Industriale

University ofParma

Via delle Scienze, Parma (Italy)

(**)	Istituto di Ingegneria

University of Ferrara

Via Saragat, Ferrara (Italy)

Description of the model

The prediction model used is based on the German Standard RLS81.

Input data:

- average hourly traffic, separated into heavy and light vehicles;

- average speed for each group;

- dimension and type of the road and of any natural and artificial obstacles.

Characteristics of the model:

- takes into account the main characteristics which influence the propagation of noise, such as obstacles, vegetation, air absorption, reflections and diffraction;

- it makes it possible to verify the noise reduction produced by barriers, taking into account their lenght;

- takes into account also the reflections produced by the opposite screens, but it is necessary that they be classified according to the Standard ZTV81, which gives directly the reduction of the reflected field in dB(A) for the typical sound spectrum of highway traffic.

The model first enables the calculation of a reference Sound Level, at 25m from the sound source, based on the traffic and road data. After this, a set of statistical values of correction factors and empirical formulae like the one proposed by Maekawa for diffraction and screen attenuation are used to compute the attenuation of sound levels at greater distances. The model takes into account mirror images of the sound source, plus an overall “reverberation” correction for multiple reflections; the prediction values obtained are generally well correlated with the experimental verification.

The model, initially simply implemented for the prediction of traffic noise at a single point, was developed by the authors so as to allow the study of altimetrically complex environments, and the automatic contour mapping of noise levels in vertical or horizontal sections, or following the land's altimetry. This procedure yielded interesting results from a representational point of view, but required a very long calculation time. Furthermore, it was necessary to build an interface to 3-D digital land altimetry representation packages.

One of the advantages of this method is the possibility of exporting the results in graphic form to the AutoCad environment, superimposing the contour lines to the plan, section or perspective representation of the site.

Description of The site and

experimental measurements

The study was carried out in a section of the A4 motorway between Venice and Trieste. In this section, the motorway goes through a built up area, containing also a school. In some cases, the buildings (including the school) were less than 5 meters away from the road. The land was altimetrically complex with a succession of woods and huge masses of carsic rock.

The aim of the study was to design noise barriers in order to guarantee the respect of the noise limits for this area type set by Italian legislation.

First of all, a study of the traffic noise level was carried out. Measurements were made at 31 points which were considered to be of particular significance to the study. At the same time, information on the traffic, including the speed of the vehicles, was gathered, and average hourly and daily traffic was calculated, divided into day and night traffic, and light and heavy vehicles. The average traffic was:

- Day time (6 am - 10 pm): 1300 light vehicles and 250 heavy vehicles/hour;

- Night time (10 pm - 6 am): 300 light vehicles and 180 heavy vehicles/hour.

These data were used to normalize each sound level to the average traffic (assumed as standard).

Contour Map of Sound Pressure Levels (computed) and Experimental Values,

before the installation of barriers

Contour Map of Sound Pressure Levels (computed) and Experimental Values,

after the installation of barriers

Vertical cross-section used for barrier calculation

Comparison of Sound Pressure Levels before and after installation of  barriers

CONCLUSIONS

- Effective, large reduction of sound pressure levels in all the points previously exceeding the limits. In some points the sound reduction appears to be even greater than required.

- In the three most critical points (1, 4 and 6), the sound reduction was respectively 15.8, 20.4 and 18.3 dBA, with a maximum deviation of 1.5 dB from the numerical previsions.

- Studying the original situation, simulating the sound propagation and designing the sound attenuating devices within an integrated framework reduces the probability of errors, minimises the overall analysis time and makes it possible to better define the sound reduction required.

- The numerical model is now being updated, to comply with the new version of RLS, namely RLS-90, and it will include the railway noise standard SCHALL-03.