# Applied Acoustics – 24 July 2015

Name & Surname:

A

B

C

D

E

F

Date of Birth:

Exercise 1 (tolerance +/- 0.5 dB)

Inside a small cabin an intensimetric measurement is performed, and the following values are found: Sound Pressure Level is equal to 80+F d, Particle Velocity Level is equal to 80+E dB, and the phase shift between pressure and velocity is 70+D degrees. Compute the Sound Intensity Level, the Energy Density Level and the Reactivity Index (RI=Ld-Li)

- Sound Intensity Level Li			                 	dB	      	(5 points)

- Energy Density Level Ld		      		             dB	       	(5 points)

- Reactivity Index RI 		     		             dB	       	(5 points)

Exercise 2 (tolerance +/- 0.5 dB)

R

S

C+10

10+E

F/10+1

An omnidirectional point source, radiating incoherent noise, is located above the reflecting floor (α=0), at an height of F/10+1 m. The Sound Power Level Lw is equal to 100+D dB.

A microphone is located at an horizontal distance of 10+E m, and at an height of C+10 m above the absorbing ground.

Determine the following values of the SPL at the microphone.

- Direct Sound SPL				                 		dB       	(5 points)

- Reflected Sound SPL				                 	dB       	(5 points)

- Direct Sound SPL				                 		dB       	(5 points)